[image: image7.png]



Instructions for Installation and Care of CF Moto Z Force Soft Door Kit
(CF-ZFORCE-SDK01)
Notice: Do not smoke inside the cab or expose the enclosure to direct flames. Cab is Flammable. 

Be sure to read these instructions thoroughly before opening and installing the enclosure. The installation is best done when the temperature is 70 degrees F. or warmer. The temperature affects the adhesive Velcro, if conditions prevent installing in warmer weather it helps to store the adhesive Velcro at room temperature prior to installation.

1. Carefully open the carton and remove the enclosure on a large flat surface that is clean and free of dirt.

2. Check over the parts included in the carton with the parts list shown below:

Soft Doors

Adhesive Velcro Hook Strips Qty            Length

18 each                                                         3"

2 each                                                           20"

Snap Screw Studs


   21 each (1 extra)
3. Begin by holding the door up to the frame and attach it using the double-sided Velcro strips sewn perpendicular to the edge of the door.  At each of the double-sided strips take one of the 3" adhesive Velcro strips and adhere it to the inside of the frame.  Then wrap the double-sided strip around it so that the adhesive strip holds it firmly in place.  The following diagrams show how it should look.

[image: image2.jpg]Double-Sided
Velcro

3 ”Adhesive

Enclosure Velero




[image: image3.jpg]3" Adhesive
Velcro

Double-Sided
Velcro

Enclosure





4. Next take the 20" adhesive Velcro strips and run it down the back of the roll cage facing towards the rear of the vehicle.  Start at the bottom and work your way up the bar.  You will see the double-sided Velcro tabs sewn along the back of the doors.  See photo below.
[image: image4.jpg]



5. Once all the Velcro is in place the last thing to do is install the screw snap studs.  NOTE: The best way to install the snap studs is to put them in the sockets in the door first.  Then push them into the plastic to make a mark.  Then pull the screw stud out and screw it into the plastic where the mark was made.  See Photo on next page.
[image: image5.jpg]



Cleaning and Storage Recommendations:
1. Before removing the Cab for storage, make sure it is very clean in order to prevent scratching of the window panels.  Clean with a soft rag or sponge using a mild liquid detergent soap and warm water solution.  Never rub the windows with a dry cloth or rag! Let the Cab dry thoroughly before attempting to roll for storage.

2. It is important that the Cab be rolled up for storage rather than folding.  Folding will cause damage to the windows!!!  Carefully roll up the Cab like the way it was when it was packed at the factory.  On a large, clean and flat surface, fold the doors over the rear panel first, then, interleave the top and windshield on top of the doors.  Begin rolling the Cab into a roll approximately 8 inches in diameter.   Put the rolled Cab back into the carton for storage at a normal room temperature.  It is best not to expose the Cab to either extreme hot or cold temperatures when storing.

If you have any questions, comments, need assistance, or are missing parts, please contact us by one of the methods below.

[image: image6.png]=4 service@seizmik.com o . @ O o @

1-866-838-3366
between 9:00AM and 5:00PM EST. www.seizmik.com/customer-support




[image: image1.png]


WARNING: This product contains a chemical known to the State of California to cause cancer. For more information, go to www.P65Warnings.ca.gov.
